Goal:
- To obtain the pose estimation of the heatpipes on the ground, based on the given bag file data. The bag file data currently consists of only point cloud & RGB image data. Identifying the heat pipes would be the first task. Pipe detection has to be reliable and accurate since there is a very small threshold of error for the robot to get on to the pipes.

Pose estimation can be carried out using either the point cloud data or rgb image. We can approach this problem using the point cloud library (PCL) and also with a neural network based detection and pose estimation. Once we have 2 approaches, we can compare the approaches and identify the best suited method to implement for the next phase.

Inputs:

- Point cloud

- Stereo Images

- Field of view Horizontal 87deg , vertical 58 deg

System Usage:

- ROS2 Humble

- Ubuntu 22.04

- GPU MAC M1/ GPU from Google Colab

Tasks

Week 1

- Setup ROS 2 environment

- Converted bag to a ROS 2 Format and did bag review to decide next steps

- Suggested using QR code but can’t do that

- Converted Stereo Images to .JPEG files (1000+) files after writing a rosnode

- Installed labelImg (Python package that has gui to label images)

- Downloaded and built RTABMAP and YOLOv7 code.

Week 2: 06/02/2023-10/02/2023

- Manual Image labeling and segregation for train and validation

- Figured out how to use GPU in VM. (not possible on m1 mac)

- Tried using mac and gpu but yolov7 doesn’t support MPS M1 Gpu’s yet. I tried updating code but that would have caused bugs and waste time.

- I then setup a Google Colab notebook to run the training model since it provides GPUs.

- The labeling was wrong due to prior labels in the package I used. So, I manually corrected the labels.

- Finally I updated the configs and ran the training. The weights will be stored in the google colab.

- Got the trained weights: yolov7-custom16/weights/best.pt

- Detection failed. Need to re-train with a better and cleaner dataset.

- val: Scanning 'data/validate/labels.cache' images and labels... 13 found, 0 missing, 0 empty, 0 corrupted: 100% 13/13 [00:00<?, ?it/s]

- Class      Images      Labels           P           R      mAP@.5  mAP@.5:.95: 100% 1/1 [00:02<00:00,  2.53s/it]

- all          13          16     0.00308        0.75      0.0033    0.000627

- Speed: 47.6/7.7/55.3 ms inference/NMS/total per 640x640 image at batch-size 32

- Re trained the model on less data. And the results were still not good.

- Taking a step back and doing more research on how to create the dataset properly.

- Investigated Yolo V3, RetinaNet, Faster R CNN, Mask R CNN, Cascade R CNN.

- Trained a yolo v3 model and got the weights. Got 99% in mAP@0.5 and prediction works.

- Downloaded the weights. Now need to run it locally on all the images and find a way to run it when playing rosbag. (WIP)

Week 3 : 11/02/2023- 17/02/2023

- Goal: Detect horizontal trails of the pipe as well and do pose estimation using camera link and base link.

- Labeled images with left, right edges and track with pipe. (total 193 images)

- Train 162 images, validate: 31 images

- Weight stored in exp 14

- For pose estimation, we have bounding boxes of left trail, right trail as well as track and pipe entry. We need to estimate pose of the bounding box/diagonal of box for trails w.r.t. Camera.

- Pose estimation libraries:

  - https://github.com/GeorgeDu/6d-object-pose-estimation#category-level

  - https://arxiv.org/pdf/2001.10609.pdf

  - https://github.com/topics/6d-pose-estimation

  - https://github.com/zubair-irshad/CenterSnap

  - Learning techniques for pose estimation are trained on LineMOD dataset.The datasets include 3D object models and training and test RGB-D images annotated with ground-truth 6D object poses and intrinsic camera parameters.

  - https://arxiv.org/pdf/2005.14502.pdf Use 3d point cloud and rgb image to extract key points, train the descriptors and create pair correspondence between 2d and 3d points to get 6 DOF Pose on new rgb images. (no open source code afaik)

- Following links work on bounding box and with 3D Model

  - https://towardsdatascience.com/geometric-deep-learning-for-pose-estimation-6af45da05922 , https://github.com/vaishak2future/posefromkeypoints

  - https://docs.opencv.org/3.4/dc/d2c/tutorial_real_time_pose.html

- The last two links (Pose from key points) and Real time pose needs following:

  - Given a Bounding box, we can create a keypoint descriptors (Cropped Image with keypoint heatmaps overlayed/ orb features)

  - We need a 3D Model of the object we are interested in. Example: https://github.com/opencv/opencv/blob/4.x/samples/cpp/tutorial_code/calib3d/real_time_pose_estimation/src/main_registration.cpp . For this, we need a .ply file of (3D Model) of the object.

  - Another way to get a model: https://arxiv.org/pdf/1703.04670.pdf (Kinect Fusion - Given RGB-D image where depth is used only for ground truth. The ground truth object pose for each image was calculated by ICP with careful manual initialization.  A complete 3D model of the gas canister was reconstructed using KinectFusion. Then, 10 keypoints were manually defined on the 3D model and projected to the images yielding ground truth keypoint locations in 2D.

- Current situation and what we are missing:

  - Given:

    - Bounding Box of the object. (i.e. 2D Image points) camera_T_pipe.

  - Task:

    - Localize the pipe i.e. find the position of the pipe in the real world. world_T_pipe.

  - To convert pixel values of the pipe in the image to a real world pose, we need to know the scale between pixel to mm i.e. camera_T_world transformation and apply PnP to find the Transformation.

    - Establish point correspondence between 2d image pixels w.r.t. 3d world points.

      - Create a 3D Model of the pipe (.ply file) , then do feature extraction and flann matching with 3D Model to get point correspondence between 2d-3d points and do PnP to get the desired result. (Explore using Kinect Fusion, but Need scan of the pipe to create this) - https://github.com/opencv/opencv/tree/4.x/samples/cpp/tutorial_code/calib3d/real_time_pose_estimation

      - Semantic keypoints: Given RGB-D image where depth is used only for ground truth. The ground truth object pose for each image was calculated by ICP with careful manual initialization.  A complete 3D model of the object was reconstructed using KinectFusion. Then, 10 keypoints were manually defined on the 3D model and projected to the images yielding ground truth keypoint locations in 2D. This is a learning based algorithm. https://towardsdatascience.com/geometric-deep-learning-for-pose-estimation-6af45da05922

      - Point correspondence between 2D Image and 3D Point cloud using keypoint descriptors for both the dataset. For training the model after keypoint extraction, we need to establish point correspondence which can be done if we have the camera's extrinsic and intrinsic parameters.  No open source code

    - This can be done manually (by physically measuring the world w.r.t. pixel) but it will be tedious and might be different for each camera. Can be error prone.

  - Another way to look at the problem is as a slam problem. Instead of finding world_T_pipe, we find map_T_pipe. Map is generated using RTABMAP and we localize the robot in the map i.e. get map_T_base_link. From bounding box, for every pixel in it we have, camera_T_pipe. We also know base_link_T_camera. So we can now get map_T_pipe = map_T_camera * camera_T_pipe.

- Blue colored algorithm implementation:

  - Building and compiling code on Ubuntu 22.04 and run on custom 3D Model to create a mesh.

  - Step 1 - Creating a 3D Textured Model

    - Input:

      - Image of the pipe

      - 3D Model of the pipe (.ply)

      - Intrinsic Parameters: color: [ 1280x720 p[648.812 360.262] f[643.008 642.201] Inverse Brown Conrady [-0.0562901 0.0673987 -0.00054218 0.000138015 -0.0220508] ]

      - depth: [ 848x480 p[424.16 239.665] f[429.381 429.381] Brown Conrady [0 0 0 0 0] ]

    - Output: yml file (done)

  - Step 2 - Pose Estimation using loaded information (model detection)

  - Problem: The initial pipes.ply file sent had 720 vertices that wasn’t possible to match and align the axis.

  - Created a new .ply file for two pipes with number of vertices to be 8 for each cylinder that means the top of the cylinder is assumed to be a square. By default, Blender (software), used 32 vertices for defining a circle but we don’t need that.

  - Using .ply, the model registration can get correspondences but the pipes are not thick enough to have a ton of orb features. This is a blocker. There are only 9 features inside the pipe and 1991 outliers.

- Come up with a new approach to improvise

  - I decided to use only lines as features since pipe has lines. I implemented a line detector with a descriptor to use as features. The detector was detecting too many straight lines.

  - I added the code to use bounding box around the pipe  (ideally this will come from yolo) and only detect line features inside the bounding box. This wasn’t helpful as well since box is still big.

  - I added code to filter the detected lines based on the length of the lines. This helps in reducing the outliers but doesn’t increase the inliners. Below ss with filtered lines

  - The output only has 2 inliners which isn’t enough for tracking.

  - For bounding box, using ORB features also doesn’t work.

  - This algorithm doesn’t seem to work for the use case of detecting pipe.

- Research new techniques, probably how to use depth information for pose estimation. (Similar to http://wiki.ros.org/find_object_2d)

  - We have depth/points i.e. sensor_msgs/PointCloud topic.

  - This algorithm needs an rgb image as well as a depth aligned image. This depth aligned image stores depth value at each pixel so the size should match and both the topics need to be in sync.

  - The algorithm uses Feature matching to detect object. Then uses the center of the detected object (bounding box) as well 3 more points inside the box (pixel points). For every pixel point, we can get XYZ coordinate using camera’s info, pixel value as well as depth (mat) image. Then we apply PnP to get the tf/pose between camera_link and object

  - To implement the same, the first step is to create a depth aligned image from point cloud points topic in the rosbag. Write a script for this.

TODO:

- Run yolov3 on the bag with new weights.

- Given 3D Cad model, use opencv pose estimation (WIP)

- Need to decide evaluation of the algorithms

  - Accuracy

  - Pose Error Translation as well as rotational

Approach

- Using a NEURAL NETWORK based object detection technique like YOLOv7 to get a bounding box with center and corners.

- Using a classical approach to detect pipes in the environment (computer vision/ pcl based) and get a pose.

Next Goal

- Need a visual slam based mapping algorithm to create a map.

  - Once we have a map, we need to convert the position from one of the two approaches to map co-ordinates for localization.